societe2#sh run

Building configuration...

Current configuration : 3259 bytes

!

! Last configuration change at 15:56:42 UTC Mon Jan 13 2025

!

version 15.8

service config

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname societe2

!

boot-start-marker

boot-end-marker

!

!

!

no aaa new-model

!

!

!

!

!

!

!

!

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

ip dhcp excluded-address 192.168.40.1

!

ip dhcp pool vlan 10

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 10.0.0.1

!

ip dhcp pool vlan 20

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 10.0.0.1

!

ip dhcp pool vlan 30

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 10.0.0.1

!

ip dhcp pool vlan 40

network 192.168.40.0 255.255.255.0

default-router 192.168.40.1

dns-server 10.0.0.1

!

ip dhcp pool vlan10

default-router 192.168.10.1

!

ip dhcp pool vlan20

default-router 192.168.20.1

!

ip dhcp pool vlan30

default-router 192.168.30.1

!

ip dhcp pool vlan40

default-router 192.168.40.1

!

!

!

ip domain name societe2.com

ip cef

no ipv6 cef

multilink bundle-name authenticated

!

!

!

license udi pid C927-4P sn FGL2348L2EW

!

!

username admin privilege 15 secret 5 $1$W9NB$loiJogdzbDgdOHspYJmPh.

!

redundancy

!

!

controller VDSL 0

!

!

!

!

!

interface ATM0

no ip address

shutdown

no atm ilmi-keepalive

!

interface Ethernet0

no ip address

shutdown

!

interface GigabitEthernet0

switchport mode trunk

no ip address

!

interface GigabitEthernet1

no ip address

!

interface GigabitEthernet2

no ip address

!

interface GigabitEthernet3

no ip address

!

interface GigabitEthernet4

ip address 10.2.1.1 255.0.0.0

ip nat outside

ip virtual-reassembly in

duplex auto

speed auto

!

interface Vlan1

no ip address

!

interface Vlan10

ip address 192.168.10.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

!

interface Vlan20

ip address 192.168.20.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

!

interface Vlan30

ip address 192.168.30.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

!

interface Vlan40

ip address 192.168.40.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

!

ip forward-protocol nd

no ip http server

no ip http secure-server

!

!

ip nat inside source static tcp 192.168.10.3 80 interface GigabitEthernet0/2 80

ip nat inside source list NAT_VLAN10 interface GigabitEthernet0/2 overload

ip nat inside source list NAT_VLAN20 interface GigabitEthernet0/2  overload

ip nat inside source list NAT_VLAN30 interface GigabitEthernet0/2  overload

ip nat inside source list NAT_VLAN40 interface GigabitEthernet0/2  overload

ip nat inside source list NAT_VLAN50 interface GigabitEthernet0/2  overload

ip route 0.0.0.0 0.0.0.0 10.0.0.1

ip ssh version 2

!

ip access-list extended NAT_VLAN10

permit ip 192.168.10.0 0.0.0.255 any

ip access-list extended NAT_VLAN20

permit ip 192.168.20.0 0.0.0.255 any

ip access-list extended NAT_VLAN30

permit ip 192.168.30.0 0.0.0.255 any

ip access-list extended NAT_VLAN40

permit ip 192.168.40.0 0.0.0.255 any

!

!

!

!

control-plane

!

!

line con 0

line 4

no activation-character

transport preferred none

transport input all

transport output all

stopbits 1

line vty 0 4

login local

transport input ssh

transport output ssh

!

scheduler allocate 20000 1000

!

end