Sucursale(config)#do sh run

Building configuration...

Current configuration : 2460 bytes

!

! Last configuration change at 15:57:54 UTC Tue Jan 14 2025

!

version 15.8

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname Sucursale

!

boot-start-marker

boot-end-marker

!

!

!

no aaa new-model

!

!

!

!

!

!

!

!

ip dhcp excluded-address 172.31.1.1

ip dhcp excluded-address 172.31.2.1

ip dhcp excluded-address 172.31.3.1

!

ip dhcp pool vlan10

network 172.31.1.0 255.255.255.0

default-router 172.31.1.1

!

ip dhcp pool vlan20

network 172.31.2.0 255.255.255.0

default-router 172.31.2.1

!

ip dhcp pool vlan30

network 172.31.3.0 255.255.255.0

default-router 172.31.3.1

!

!

!

ip cef

no ipv6 cef

multilink bundle-name authenticated

!

!

!

license udi pid C927-4P sn FGL2348L2EW

!

!

!

redundancy

!

!

controller VDSL 0

!

!

!

!

!

interface Tunnel1

ip address 192.168.3.2 255.255.255.0

tunnel source GigabitEthernet1

tunnel destination 10.2.1.1

!

interface ATM0

no ip address

shutdown

no atm ilmi-keepalive

!

interface Ethernet0

no ip address

shutdown

!

interface GigabitEthernet0

switchport trunk allowed vlan 1-1005

switchport mode trunk

no ip address

!

interface GigabitEthernet1

no ip address

!

interface GigabitEthernet2

no ip address

!

interface GigabitEthernet3

no ip address

!

interface GigabitEthernet4

ip address 10.2.2.1 255.0.0.0

ip nat outside

ip virtual-reassembly in

ip ospf 1 area 0

duplex auto

speed auto

!

interface Vlan1

no ip address

!

router ospf 1

network 172.31.1.0 0.0.0.255 area 0

network 172.31.2.0 0.0.0.255 area 0

network 172.31.3.0 0.0.0.255 area 0

network 172.31.4.0 0.0.0.255 area 0

network 172.31.5.0 0.0.0.255 area 0

!

ip forward-protocol nd

no ip http server

no ip http secure-server

!

!

ip nat inside source list NAT_VLAN10 interface GigabitEthernet4 overload

ip nat inside source list NAT_VLAN20 interface GigabitEthernet4 overload

ip nat inside source list NAT_VLAN30 interface GigabitEthernet4 overload

ip route 0.0.0.0 0.0.0.0 10.0.0.1

!

ip access-list extended NAT_VLAN10

permit ip 172.31.1.0 0.0.0.255 any

ip access-list extended NAT_VLAN20

permit ip 172.31.2.0 0.0.0.255 any

ip access-list extended NAT_VLAN30

permit ip 172.31.3.0 0.0.0.255 any

!

!

!

tftp-server flash:/firmware/vadsl_module_img.bin

!

control-plane

!

!

line con 0

line 4

no activation-character

transport preferred none

transport input all

transport output all

stopbits 1

line vty 0 4

login

transport input none

!

scheduler allocate 20000 1000

!

end