routeur:

Router#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#do sh run

Building configuration...

Current configuration : 2455 bytes

!

! Last configuration change at 20:46:57 UTC Tue Jan 14 2025

!

version 15.6

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname Router

!

boot-start-marker

boot-end-marker

!

!

!

no aaa new-model

ethernet lmi ce

!

!

!

!

!

!

!

!

!

!

!

!

ip cef

no ipv6 cef

multilink bundle-name authenticated

!

!

!

!

license udi pid CISCO2911/K9 sn FCZ211840CK

!

!

!

redundancy

!

!

!

!

!

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/0

ip address 192.168.1.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

ip ospf 1 area 0

duplex auto

speed auto

!

interface GigabitEthernet0/1

ip address 192.168.2.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

ip ospf 1 area 0

duplex auto

speed auto

!

interface GigabitEthernet0/2

ip address 10.2.1.1 255.0.0.0

ip nat outside

ip virtual-reassembly in

duplex auto

speed auto

!

interface Serial0/0/0

no ip address

shutdown

!

interface Serial0/0/1

no ip address

shutdown

clock rate 2000000

!

router ospf 1

redistribute connected subnets

network 192.168.1.0 0.0.0.255 area 0

network 192.168.2.0 0.0.0.255 area 0

network 192.168.10.0 0.0.0.255 area 0

network 192.168.20.0 0.0.0.255 area 0

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip nat inside source list NAT_VLAN10 interface GigabitEthernet0/2 overload

ip nat inside source list NAT_VLAN20 interface GigabitEthernet0/2 overload

ip nat inside source list NAT_VLAN30 interface GigabitEthernet0/2 overload

ip nat inside source list NAT_VLAN40 interface GigabitEthernet0/2 overload

ip nat inside source list NAT_VLAN50 interface GigabitEthernet0/2 overload

ip nat inside source static tcp 192.168.10.3 80 interface GigabitEthernet0/2 80

ip route 0.0.0.0 0.0.0.0 10.0.0.1

!

ip access-list extended NAT_VLAN10

permit ip 192.168.10.0 0.0.0.255 any

ip access-list extended NAT_VLAN20

permit ip 192.168.20.0 0.0.0.255 any

ip access-list extended NAT_VLAN30

permit ip 192.168.30.0 0.0.0.255 any

ip access-list extended NAT_VLAN40

permit ip 192.168.40.0 0.0.0.255 any

permit ip 192.168.50.0 0.0.0.255 any

ip access-list extended NAT_VLAN50

!

!

!

!

control-plane

!

!

!

line con 0

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport output pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

login

transport input none

!

scheduler allocate 20000 1000

!

end

switch niveau trois 2:

Siege2(config)#do sh run

Building configuration...

Current configuration : 3534 bytes

!

version 12.2

no service pad

no service timestamps debug uptime

no service timestamps log uptime

no service password-encryption

!

hostname Siege2

!

boot-start-marker

boot-end-marker

!

!

!

!

no aaa new-model

system mtu routing 1500

ip routing

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

ip dhcp excluded-address 192.168.40.1

ip dhcp excluded-address 192.168.50.1

!

ip dhcp pool vlan10

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 10.0.0.1

!

ip dhcp pool vlan20

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 10.0.0.1

!

ip dhcp pool vlan30

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 10.0.0.1

!

ip dhcp pool vlan40

network 192.168.40.0 255.255.255.0

default-router 192.168.40.1

dns-server 10.0.0.1

!

ip dhcp pool vlan50

network 192.168.50.0 255.255.255.0

default-router 192.168.50.1

dns-server 10.0.0.1

!

!

!

!

!

license boot level ipservices

spanning-tree mode pvst

spanning-tree extend system-id

!

!

!

!

vlan internal allocation policy ascending

!

ip ssh version 2

!

!

!

interface FastEthernet0

no ip address

no ip route-cache cef

no ip route-cache

!

interface GigabitEthernet0/1

no switchport

ip address 192.168.2.2 255.255.255.0

ip ospf 1 area 0

!

interface GigabitEthernet0/2

switchport trunk allowed vlan 10,20,30,40,50

!

interface GigabitEthernet0/3

!

interface GigabitEthernet0/4

!

interface GigabitEthernet0/5

!

interface GigabitEthernet0/6

!

interface GigabitEthernet0/7

!

interface GigabitEthernet0/8

!

interface GigabitEthernet0/9

!

interface GigabitEthernet0/10

!

interface GigabitEthernet0/11

!

interface GigabitEthernet0/12

!

interface GigabitEthernet0/13

!

interface GigabitEthernet0/14

!

interface GigabitEthernet0/15

!

interface GigabitEthernet0/16

!

interface GigabitEthernet0/17

!

interface GigabitEthernet0/18

!

interface GigabitEthernet0/19

!

interface GigabitEthernet0/20

!

interface GigabitEthernet0/21

!

interface GigabitEthernet0/22

!

interface GigabitEthernet0/23

!

interface GigabitEthernet0/24

!

interface GigabitEthernet1/1

!

interface GigabitEthernet1/2

!

interface GigabitEthernet1/3

!

interface GigabitEthernet1/4

!

interface TenGigabitEthernet1/1

!

interface TenGigabitEthernet1/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

ip address 192.168.10.253 255.255.255.0

standby 10 ip 192.168.10.1

standby 10 priority 110

standby 10 preempt

!

interface Vlan20

ip address 192.168.20.254 255.255.255.0

standby 20 ip 192.168.20.1

standby 20 priority 110

standby 20 preempt

!

interface Vlan30

ip address 192.168.30.254 255.255.255.0

standby 30 ip 192.168.30.1

standby 30 priority 110

standby 30 preempt

!

interface Vlan40

ip address 192.168.40.254 255.255.255.0

standby 40 ip 192.168.40.1

standby 40 priority 110

standby 40 preempt

!

interface Vlan50

ip address 192.168.50.254 255.255.255.0

standby 50 ip 192.168.50.1

standby 50 priority 110

standby 50 preempt

!

router ospf 1

log-adjacency-changes

network 192.168.1.0 0.0.0.255 area 0

network 192.168.10.0 0.0.0.255 area 0

network 192.168.20.0 0.0.0.255 area 0

network 192.168.30.0 0.0.0.255 area 0

network 192.168.40.0 0.0.0.255 area 0

network 192.168.50.0 0.0.0.255 area 0

!

ip classless

ip route 0.0.0.0 0.0.0.0 10.0.0.1

ip route 0.0.0.0 0.0.0.0 192.168.2.1

!

ip http server

ip http secure-server

!

ip sla enable reaction-alerts

!

!

!

line con 0

line vty 0 4

login

line vty 5 15

login

!

end

switch niveau 3 1:
Siege1(config)#do sh run

Building configuration...

Current configuration : 5274 bytes

!

version 12.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname Siege1

!

boot-start-marker

boot-end-marker

!

!

!

!

no aaa new-model

system mtu routing 1500

ip routing

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

ip dhcp excluded-address 192.168.40.1

ip dhcp excluded-address 192.168.50.1

ip dhcp excluded-address 192.168.10.2

ip dhcp excluded-address 192.168.10.5

!

ip dhcp pool vlan10

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 10.0.0.1

!

ip dhcp pool vlan20

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 10.0.0.1

!

ip dhcp pool vlan30

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 10.0.0.1

!

ip dhcp pool vlan40

network 192.168.40.0 255.255.255.0

default-router 192.168.40.1

dns-server 10.0.0.1

!

ip dhcp pool vlan50

network 192.168.50.0 255.255.255.0

default-router 192.168.50.1

dns-server 10.0.0.1

!

!

!

!

crypto pki trustpoint TP-self-signed-2723487488

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-2723487488

revocation-check none

rsakeypair TP-self-signed-2723487488

!

!

crypto pki certificate chain TP-self-signed-2723487488

certificate self-signed 01

3082023F 308201A8 A0030201 02020101 300D0609 2A864886 F70D0101 04050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 32373233 34383734 3838301E 170D3933 30333031 30303031

35385A17 0D323030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D32 37323334

38373438 3830819F 300D0609 2A864886 F70D0101 01050003 818D0030 81890281

8100E6CE DB91361E 5A3DC5F2 02E32A02 72C965D7 5CAED489 A1F40562 B8B80E8D

E6AABD23 6EABD0DE F257B643 76AC9E0F E9E56715 AB85D697 5502FE85 0491099C

6B2AF92A FA5FC2CC 3ABCC54D 46903D7E 5FA1E265 F0DCF092 C7E65FCD 766E361C

07C6B835 9A96F78C 590CC14F ACB64A46 A1AAA355 65712D6A 654C44F4 41482B7C

AA670203 010001A3 67306530 0F060355 1D130101 FF040530 030101FF 30120603

551D1104 0B300982 07536965 6765312E 301F0603 551D2304 18301680 14718B18

CBA508C9 725D1258 02EBED5C 584D897E 77301D06 03551D0E 04160414 718B18CB

A508C972 5D125802 EBED5C58 4D897E77 300D0609 2A864886 F70D0101 04050003

818100B4 2D7B11D3 60E38949 1C2B0205 F56A67A5 F24D383E 858D812B 2C209A3B

6889662D 124FF9E5 F03F86C3 277F0F69 664796F6 4A0CE0AB 32A8486C C5E643A8

B249A318 21CFECDC D8AB9226 435C3A85 62B362C6 662AA1D8 E23D2FBF 1C046147

472A8049 047D4D9C A9C06C43 DC849F60 6043E413 86143EA6 22915506 D49CD63B F4F603

quit

license boot level ipservices

spanning-tree mode pvst

spanning-tree extend system-id

!

!

!

!

vlan internal allocation policy ascending

!

ip ssh version 2

!

!

!

interface FastEthernet0

no ip address

no ip route-cache cef

no ip route-cache

no ip mroute-cache

!

interface GigabitEthernet0/1

no switchport

ip address 192.168.1.2 255.255.255.0

ip ospf 1 area 0

!

interface GigabitEthernet0/2

switchport trunk allowed vlan 10,20,30,40,50

!

interface GigabitEthernet0/3

!

interface GigabitEthernet0/4

!

interface GigabitEthernet0/5

!

interface GigabitEthernet0/6

!

interface GigabitEthernet0/7

!

interface GigabitEthernet0/8

!

interface GigabitEthernet0/9

!

interface GigabitEthernet0/10

!

interface GigabitEthernet0/11

!

interface GigabitEthernet0/12

!

interface GigabitEthernet0/13

!

interface GigabitEthernet0/14

!

interface GigabitEthernet0/15

!

interface GigabitEthernet0/16

!

interface GigabitEthernet0/17

!

interface GigabitEthernet0/18

!

interface GigabitEthernet0/19

!

interface GigabitEthernet0/20

!

interface GigabitEthernet0/21

!

interface GigabitEthernet0/22

!

interface GigabitEthernet0/23

!

interface GigabitEthernet0/24

!

interface GigabitEthernet1/1

!

interface GigabitEthernet1/2

!

interface GigabitEthernet1/3

!

interface GigabitEthernet1/4

!

interface TenGigabitEthernet1/1

!

interface TenGigabitEthernet1/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

ip address 192.168.10.254 255.255.255.0

standby 10 ip 192.168.10.1

standby 10 priority 110

standby 10 preempt

!

interface Vlan20

ip address 192.168.20.253 255.255.255.0

standby 20 ip 192.168.21.1

standby 20 priority 110

standby 20 preempt

!

interface Vlan30

ip address 192.168.30.253 255.255.255.0

standby 30 ip 192.168.31.1

standby 30 priority 110

standby 30 preempt

!

interface Vlan40

ip address 192.168.40.253 255.255.255.0

standby 40 ip 192.168.40.1

standby 40 priority 110

standby 40 preempt

!

interface Vlan50

ip address 192.168.50.253 255.255.255.0

standby 50 ip 192.168.50.1

standby 50 priority 110

standby 50 preempt

!

router ospf 1

log-adjacency-changes

network 192.168.1.0 0.0.0.255 area 0

network 192.168.10.0 0.0.0.255 area 0

network 192.168.20.0 0.0.0.255 area 0

network 192.168.30.0 0.0.0.255 area 0

network 192.168.40.0 0.0.0.255 area 0

network 192.168.50.0 0.0.0.255 area 0

!

ip classless

ip route 0.0.0.0 0.0.0.0 10.0.0.1

ip route 0.0.0.0 0.0.0.0 192.168.1.1

!

ip http server

ip http secure-server

!

ip sla enable reaction-alerts

!

!

!

line con 0

line vty 0 4

login

line vty 5 15

login

!

end

switch normal:
spanning-tree extend system-id

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/2

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/3

!

interface FastEthernet0/4

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk allowed vlan 10,20

switchport mode trunk

!

interface GigabitEthernet0/2

switchport trunk allowed vlan 10,20

switchport mode trunk

!

interface Vlan1

no ip address

shutdown

!

!

!

!

line con 0

!

line vty 0 4

login

line vty 5 15

login

!

!

!

!

end

no ip dhcp excluded-address 192.168.10.1

no ip dhcp excluded-address 192.168.20.1

no ip dhcp excluded-address 192.168.30.1

no ip dhcp excluded-address 192.168.40.1

no ip dhcp excluded-address 192.168.50.1

routeur succursale :

Sucursale(config-if)#do sh run

Building configuration...

Current configuration : 2563 bytes

!

! Last configuration change at 07:28:07 UTC Wed Jan 15 2025

!

version 15.8

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname Sucursale

!

boot-start-marker

boot-end-marker

!

!

!

no aaa new-model

!

!

!

!

!

!

!

!

ip dhcp excluded-address 172.31.1.1

ip dhcp excluded-address 172.31.2.1

ip dhcp excluded-address 172.31.3.1

!

ip dhcp pool vlan10

network 172.31.1.0 255.255.255.0

default-router 172.31.1.1

!

ip dhcp pool vlan20

network 172.31.2.0 255.255.255.0

default-router 172.31.2.1

!

ip dhcp pool vlan30

network 172.31.3.0 255.255.255.0

default-router 172.31.3.1

!

!

!

ip cef

no ipv6 cef

multilink bundle-name authenticated

!

!

!

license udi pid C927-4P sn FGL2348L2EW

!

!

!

redundancy

!

!

controller VDSL 0

!

!

!

!

!

interface Tunnel1

ip address 192.168.3.2 255.255.255.252

ip mtu 1400

ip tcp adjust-mss 1360

tunnel source 10.2.2.1

tunnel destination 10.2.1.1

!

interface ATM0

no ip address

shutdown

no atm ilmi-keepalive

!

interface Ethernet0

no ip address

shutdown

!

interface GigabitEthernet0

switchport trunk allowed vlan 1-1005

switchport mode trunk

no ip address

!

interface GigabitEthernet1

no ip address

!

interface GigabitEthernet2

no ip address

!

interface GigabitEthernet3

no ip address

!

interface GigabitEthernet4

ip address 10.2.2.1 255.0.0.0

ip nat outside

ip virtual-reassembly in

duplex auto

speed auto

!

interface Vlan1

no ip address

!

interface Vlan10

ip address 172.31.1.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

!

interface Vlan20

ip address 172.31.2.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

!

interface Vlan30

ip address 172.31.3.1 255.255.255.0

ip nat inside

ip virtual-reassembly in

!

ip forward-protocol nd

no ip http server

no ip http secure-server

!

!

ip nat inside source list NAT_VLAN10 interface GigabitEthernet4 overload

ip nat inside source list NAT_VLAN20 interface GigabitEthernet4 overload

ip nat inside source list NAT_VLAN30 interface GigabitEthernet4 overload

ip route 0.0.0.0 0.0.0.0 10.0.0.1

!

ip access-list extended NAT_VLAN10

permit ip 172.31.1.0 0.0.0.255 any

ip access-list extended NAT_VLAN20

permit ip 172.31.2.0 0.0.0.255 any

ip access-list extended NAT_VLAN30

permit ip 172.31.3.0 0.0.0.255 any

!

!

!

tftp-server flash:/firmware/vadsl_module_img.bin

!

control-plane

!

!

line con 0

line 4

no activation-character

transport preferred none

transport input all

transport output all

stopbits 1

line vty 0 4

login

transport input none

!

scheduler allocate 20000 1000

!

end

interface Tunnel0

ip address 2.2.2.1 255.255.255.0

tunnel source GigabitEthernet0/2

tunnel destination 10.2.2.1

tunnel mode gre ip

no shut