interface FastEthernet0/24

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

ip address 192.168.10.254 255.255.255.0

!

interface Vlan20

ip address 192.168.20.254 255.255.255.0

!

interface Vlan30

ip address 192.168.30.254 255.255.255.0

!

ip classless

##### en

##### conf t

# hostname sw-France

ip domain-name societea1.lan

crypto key generate rsa general-keys modulus 1024

ip ssh version 2

aaa new-model

aaa authentication login default local

aaa authorization exec default local

username admin secret admin-swfr-Progtr00

enable secret en-swfr-Progtr00

### ip dhcp excluded-address 192.168.10.1 192.168.10.10

ip dhcp excluded-address 192.168.20.1 192.168.20.10

ip dhcp excluded-address 192.168.30.1 192.168.30.10

!

### ip dhcp pool vPool10

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 192.168.20.10

!

ip dhcp pool vPool20

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 192.168.20.10

!

ip dhcp pool vPool30

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 192.168.20.10

### interface range GigabitEthernet0/1-9

switchport access vlan 10

switchport mode access

no sh

### interface range GigabitEthernet0/10-19

switchport access vlan 20

switchport mode access

no sh

### interface range GigabitEthernet0/20-23

switchport access vlan 30

switchport mode access

no sh

### interface GigabitEthernet0/24

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

ip address 192.168.10.5 255.255.255.0

!

interface Vlan20

ip address 192.168.20.5 255.255.255.0

!

interface Vlan30

ip address 192.168.30.5 255.255.255.0

!

en

conf t

# hostname sw-Angleterre

ip domain-name societea1.lan

crypto key generate rsa general-keys modulus 1024

ip ssh version 2

aaa new-model

aaa authentication login default local

aaa authorization exec default local

username admin secret admin-swan-Progtr00

enable secret en-swan-Progtr00

ip dhcp excluded-address 192.168.11.1 192.168.11.10

ip dhcp excluded-address 192.168.21.1 192.168.21.10

ip dhcp excluded-address 192.168.31.1 192.168.31.10

!

ip dhcp pool vPool10

network 192.168.11.0 255.255.255.0

default-router 192.168.11.1

dns-server 192.168.20.10

!

ip dhcp pool vPool20

network 192.168.21.0 255.255.255.0

default-router 192.168.21.1

dns-server 192.168.20.10

!

ip dhcp pool vPool30

network 192.168.31.0 255.255.255.0

default-router 192.168.31.1

dns-server 192.168.20.10

interface range GigabitEthernet0/1-9

switchport access vlan 10

switchport mode access

no sh

interface range GigabitEthernet0/10-19

switchport access vlan 20

switchport mode access

no sh

interface range GigabitEthernet0/20-23

switchport access vlan 30

switchport mode access

no sh

interface GigabitEthernet0/24

switchport trunk encapsulation dot1q

switchport mode trunk

no sh

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

ip address 192.168.11.5 255.255.255.0

!

interface Vlan20

ip address 192.168.21.5 255.255.255.0

!

interface Vlan30

ip address 192.168.31.5 255.255.255.0

!