Routeur-siege(config-if)#do sh run

Building configuration...

Current configuration : 1919 bytes

!

upgrade fpd auto

version 12.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname Routeur-siege

!

boot-start-marker

boot-end-marker

!

logging message-counter syslog

!

no aaa new-model

ip source-route

no ip icmp rate-limit unreachable

ip cef

!

!

!

!

no ip domain lookup

no ipv6 cef

!

multilink bundle-name authenticated

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

archive

log config

hidekeys

!

!

!

!

!

ip tcp synwait-time 5

!

!

!

!

interface FastEthernet0/0

ip address 192.168.1.1 255.255.255.0

ip ospf 1 area 0

duplex half

!

interface FastEthernet1/0

no ip address

duplex half

!

interface GigabitEthernet2/0

ip address 10.2.1.1 255.0.0.0

negotiation auto

!

interface GigabitEthernet3/0

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet4/0

no ip address

shutdown

negotiation auto

!

router ospf 1

log-adjacency-changes

redistribute connected subnets

network 192.168.1.0 0.0.0.255 area 0

network 192.168.2.0 0.0.0.255 area 0

network 192.168.10.0 0.0.0.255 area 0

network 192.168.20.0 0.0.0.255 area 0

network 192.168.30.0 0.0.0.255 area 0

!

ip forward-protocol nd

no ip http server

no ip http secure-server

!

!

ip nat inside source list NAT_VLAN10 interface GigabitEthernet2/0 overload

ip nat inside source list NAT_VLAN20 interface GigabitEthernet2/0 overload

ip nat inside source list NAT_VLAN30 interface GigabitEthernet2/0 overload

!

ip access-list extended NAT_VLAN10

permit ip 192.168.10.0 0.0.0.255 any

ip access-list extended NAT_VLAN20

permit ip 192.168.20.0 0.0.0.255 any

ip access-list extended NAT_VLAN30

permit ip 192.168.30.0 0.0.0.255 any

!

no cdp log mismatch duplex

!

!

!

!

!

!

control-plane

!

!

!

!

!

!

!

gatekeeper

shutdown

!

!

line con 0

exec-timeout 0 0

privilege level 15

logging synchronous

stopbits 1

line aux 0

exec-timeout 0 0

privilege level 15

logging synchronous

stopbits 1

line vty 0 4

login

!

end

Siege1(config-if)#do sh run

Building configuration...

Current configuration : 4219 bytes

!

version 12.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

no service dhcp

!

hostname Siege1

!

boot-start-marker

boot-end-marker

!

!

no aaa new-model

memory-size iomem 5

no ip icmp rate-limit unreachable

ip cef

!

!

no ip dhcp use vrf connected

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

!

ip dhcp pool vlan10

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 10.0.0.1

!

ip dhcp pool vlan20

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 10.0.0.1

!

ip dhcp pool vlan30

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 10.0.0.1

!

!

no ip domain lookup

!

multilink bundle-name authenticated

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

macro name add_vlan

end

vlan database

vlan $v

exit

@

macro name del_vlan

end

vlan database

no vlan $v

exit

@

!

vtp file nvram:vlan.dat

archive

log config

hidekeys

!

!

!

vlan internal allocation policy ascending

!

ip tcp synwait-time 5

!

!

!

!

interface FastEthernet0/0

description *** Unused for Layer2 EtherSwitch ***

no ip address

shutdown

duplex auto

speed auto

!

interface FastEthernet0/1

description *** Unused for Layer2 EtherSwitch ***

no ip address

shutdown

duplex auto

speed auto

!

interface FastEthernet1/0

switchport access vlan 10

duplex full

speed 100

!

interface FastEthernet1/1

switchport access vlan 20

duplex full

speed 100

!

interface FastEthernet1/2

switchport access vlan 30

duplex full

speed 100

!

interface FastEthernet1/3

no switchport

ip address 192.168.1.2 255.255.255.0

ip ospf 1 area 0

duplex full

speed 100

!

interface FastEthernet1/4

duplex full

speed 100

!

interface FastEthernet1/5

duplex full

speed 100

!

interface FastEthernet1/6

duplex full

speed 100

!

interface FastEthernet1/7

duplex full

speed 100

!

interface FastEthernet1/8

duplex full

speed 100

!

interface FastEthernet1/9

duplex full

speed 100

!

interface FastEthernet1/10

duplex full

speed 100

!

interface FastEthernet1/11

duplex full

speed 100

!

interface FastEthernet1/12

duplex full

speed 100

!

interface FastEthernet1/13

duplex full

speed 100

!

interface FastEthernet1/14

duplex full

speed 100

!

interface FastEthernet1/15

duplex full

speed 100

!

interface FastEthernet2/0

no ip address

shutdown

duplex auto

speed auto

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

ip address 192.168.10.254 255.255.255.0

standby 10 ip 192.168.10.1

standby 10 priority 110

standby 10 preempt

!

interface Vlan20

ip address 192.168.20.253 255.255.255.0

standby 20 ip 192.168.20.1

standby 20 priority 110

standby 20 preempt

!

interface Vlan30

ip address 192.168.30.253 255.255.255.0

standby 30 ip 192.168.30.1

standby 30 priority 110

standby 30 preempt

!

router ospf 1

log-adjacency-changes

network 192.168.1.0 0.0.0.255 area 0

network 192.168.10.0 0.0.0.255 area 0

network 192.168.20.0 0.0.0.255 area 0

network 192.168.30.0 0.0.0.255 area 0

!

ip forward-protocol nd

ip route 0.0.0.0 0.0.0.0 10.0.0.1

ip route 0.0.0.0 0.0.0.0 192.168.1.1

!

!

no ip http server

no ip http secure-server

!

no cdp log mismatch duplex

!

!

!

!

!

!

control-plane

!

!

!

!

!

!

!

!

!

banner exec ^C

***************************************************************

This is a normal Router with a Switch module inside (NM-16ESW)

It has been pre-configured with hard-coded speed and duplex

To create vlans use the command "vlan database" in exec mode

After creating all desired vlans use "exit" to apply the config

To view existing vlans use the command "show vlan-switch brief"

Alias(exec) 	: vl   - "show vlan-switch brief" command

Alias(configure): va X - macro to add vlan X

Alias(configure): vd X - macro to delete vlan X

***************************************************************

^C

alias configure va macro global trace add_vlan $v

alias configure vd macro global trace del_vlan $v

alias exec vl show vlan-switch brief

!

line con 0

exec-timeout 0 0

privilege level 15

logging synchronous

line aux 0

exec-timeout 0 0

privilege level 15

logging synchronous

line vty 0 4

login

!

!

end