Routeur 1

enable

configure terminal

hostname Router1

interface Tunnel0

ip address 192.168.100.1 255.255.255.252

tunnel source GigabitEthernet0/1

tunnel destination 80.0.0.1

exit

ip route 192.168.11.0 255.255.255.0 192.168.100.2

ip route 192.168.21.0 255.255.255.0 192.168.100.2

ip route 192.168.31.0 255.255.255.0 192.168.100.2

router ospf 1

network 192.168.100.0 0.0.0.3 area 0

network 192.168.11.0 0.0.0.255 area 0

network 192.168.21.0 0.0.0.255 area 0

network 192.168.31.0 0.0.0.255 area 0

exit

Routeur 2

enable

configure terminal

hostname Router2

interface GigabitEthernet0/0

ip address 50.0.0.2 255.0.0.0

no shutdown

exit

interface GigabitEthernet0/1

ip address 60.0.0.2 255.0.0.0

no shutdown

exit

ip route 80.0.0.1 255.255.255.255 60.0.0.1

ip route 50.0.0.1 255.255.255.255 50.0.0.1

router ospf 1

network 50.0.0.0 0.255.255.255 area 0

network 60.0.0.0 0.255.255.255 area 0

exit

Routeur 3

enable

configure terminal

hostname Router3

interface Tunnel0

ip address 192.168.100.2 255.255.255.252

tunnel source GigabitEthernet0/1

tunnel destination 50.0.0.1

exit

ip route 192.168.11.0 255.255.255.0 192.168.100.1

ip route 192.168.21.0 255.255.255.0 192.168.100.1

ip route 192.168.31.0 255.255.255.0 192.168.100.1

router ospf 1

network 192.168.100.0 0.0.0.3 area 0

network 192.168.11.0 0.0.0.255 area 0

network 192.168.21.0 0.0.0.255 area 0

network 192.168.31.0 0.0.0.255 area 0

exit

Switch 1

enable

configure terminal

hostname Switch1

ip routing

interface Vlan10

ip address 192.168.11.1 255.255.255.0

no shutdown

interface Vlan20

ip address 192.168.21.1 255.255.255.0

no shutdown

interface Vlan30

ip address 192.168.31.1 255.255.255.0

no shutdown

interface Vlan40

ip address 70.0.0.3 255.0.0.0

no shutdown

ip dhcp excluded-address 192.168.11.1 192.168.11.10

ip dhcp excluded-address 192.168.21.1 192.168.21.10

ip dhcp excluded-address 192.168.31.1 192.168.31.10

ip dhcp pool VLAN10

network 192.168.11.0 255.255.255.0

default-router 192.168.11.1

dns-server 8.8.8.8

ip dhcp pool VLAN20

network 192.168.21.0 255.255.255.0

default-router 192.168.21.1

dns-server 8.8.8.8

ip dhcp pool VLAN30

network 192.168.31.0 255.255.255.0

default-router 192.168.31.1

dns-server 8.8.8.8

interface FastEthernet0/1

switchport mode access

switchport access vlan 10

no shutdown

interface FastEthernet0/2

switchport mode access

switchport access vlan 20

no shutdown

interface FastEthernet0/3

switchport mode access

switchport access vlan 30

no shutdown

interface FastEthernet0/4

switchport mode access

switchport access vlan 40

no shutdown

Switch 2

enable

configure terminal

hostname Switch2

ip routing

interface Vlan10

ip address 192.168.11.254 255.255.255.0

no shutdown

interface Vlan20

ip address 192.168.21.254 255.255.255.0

no shutdown

interface Vlan30

ip address 192.168.31.254 255.255.255.0

no shutdown

ip dhcp excluded-address 192.168.11.254 192.168.11.10

ip dhcp excluded-address 192.168.21.254 192.168.21.10

ip dhcp excluded-address 192.168.31.254 192.168.31.10

ip dhcp pool VLAN10

network 192.168.11.0 255.255.255.0

default-router 192.168.11.254

dns-server 8.8.8.8

ip dhcp pool VLAN20

network 192.168.21.0 255.255.255.0

default-router 192.168.21.254

dns-server 8.8.8.8

ip dhcp pool VLAN30

network 192.168.31.0 255.255.255.0

default-router 192.168.31.254

dns-server 8.8.8.8

interface FastEthernet0/1

switchport mode access

switchport access vlan 10

no shutdown

interface FastEthernet0/2

switchport mode access

switchport access vlan 20

no shutdown

interface FastEthernet0/3

switchport mode access

switchport access vlan 30

no shutdown

pkill firefox

sudo rm -rf ~/.cache/mozilla/firefox/* ~/.mozilla/firefox/*/cache*

sudo chown -R $USER:$USER ~/.cache/mozilla/firefox ~/.mozilla/firefox

rm -rf ~/.cache/mozilla/firefox/* ~/.mozilla/firefox/*/cache*

lsattr -R ~/.cache/mozilla/firefox ~/.mozilla/firefox

sudo chattr -R -i ~/.cache/mozilla/firefox ~/.mozilla/firefox

rm -rf ~/.cache/mozilla/firefox/* ~/.mozilla/firefox/*/cache*

rm -rf ~/snap/firefox/common/.cache/*

firefox &