##### en

##### conf t

# hostname R3-ESPAGNE

### interface GigabitEthernet0/0

ip address 172.16.10.1 255.255.255.0

no sh

ip nat inside

ip virtual-reassembly in

duplex auto

speed auto

!

### interface GigabitEthernet0/1

ip address 172.16.20.1 255.255.255.0

ip nat inside

no sh

ip virtual-reassembly in

duplex auto

speed auto

!

### interface GigabitEthernet0/2

ip address dhcp

no sh

ip nat outside

ip virtual-reassembly in

duplex auto

speed auto

### router ospf 1

redistribute connected subnets

network 172.16.10.0 0.0.0.255 area 0

network 172.16.20.0 0.0.0.255 area 0

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

### ip nat inside source list 1 interface GigabitEthernet0/2 overload

### ip route 0.0.0.0 0.0.0.0 10.0.0.1

!

!

!

access-list 1 permit any

!

control-plane

!

##### en

##### conf t

# hostname R2-ANGLETERRE

### interface GigabitEthernet0/1

ip address 172.16.20.2 255.255.255.0

no sh

duplex auto

speed auto

!

### interface GigabitEthernet0/2

ip address 172.16.35.1 255.255.255.0

no sh

duplex auto

speed auto

!

### interface Serial0/0/0

no ip address

shutdown

!

### interface Serial0/0/1

no ip address

shutdown

clock rate 2000000

!

### router ospf 1

redistribute connected subnets

network 172.16.20.0 0.0.0.255 area 0

network 172.16.35.0 0.0.0.255 area 0

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 0.0.0.0 0.0.0.0 172.16.20.1

!

!

!

!

##### en

##### conf t

# hostname R1-FRANCE

### interface GigabitEthernet0/0

ip address 172.16.10.2 255.255.255.0

no sh

duplex auto

speed auto

!

### interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

!

### interface GigabitEthernet0/2

ip address 172.16.30.1 255.255.255.0

no sh

duplex auto

speed auto

!

interface Serial0/0/0

no ip address

shutdown

!

interface Serial0/0/1

no ip address

shutdown

clock rate 2000000

!

### router ospf 1

redistribute connected subnets

network 172.16.10.0 0.0.0.255 area 0

network 172.16.30.0 0.0.0.255 area 0

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

### ip route 0.0.0.0 0.0.0.0 172.16.10.1

ip route 192.168.100.0 255.255.255.0 172.16.30.2

!

!

!

no access-list OUTSIDE_IN extended permit icmp any any

no access-list VLAN10_IN extended permit icmp 192.168.10.0 255.255.255.0 any

no access-list VLAN10_IN extended deny icmp any 192.168.10.0 255.255.255.0

no access-list VLAN20_30_IN extended deny ip 192.168.20.0 255.255.255.0 192.168.30.0 255.255.255.0

no access-list VLAN20_30_IN extended deny ip 192.168.30.0 255.255.255.0 192.168.20.0 255.255.255.0

!

!

no access-group OUTSIDE_IN in interface outside

no access-group VLAN20_30_IN in interface inside