L3-Siege :

L3-Sige>en

L3-Sige#sh run

Building configuration...

Current configuration : 2598 bytes

!

version 12.2(37)SE1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname L3-Sige

!

!

!

ip dhcp excluded-address 192.168.10.5

ip dhcp excluded-address 192.168.10.1

ip dhcp excluded-address 192.168.20.1

ip dhcp excluded-address 192.168.30.1

!

ip dhcp pool Vlan10

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 192.168.10.5

ip dhcp pool Vlan20

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 192.168.10.5

ip dhcp pool Vlan30

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 192.168.10.5

!

!

ip routing

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/2

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/3

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/4

no switchport

ip address 192.168.1.2 255.255.255.0

ip ospf 1 area 0

duplex auto

speed auto

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk allowed vlan 10,20,30

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

mac-address 0006.2aec.7001

ip address 192.168.10.254 255.255.255.0

!

interface Vlan20

mac-address 0006.2aec.7002

ip address 192.168.20.254 255.255.255.0

standby 20 ip 192.168.20.1

standby 20 priority 110

standby 20 preempt

!

interface Vlan30

mac-address 0006.2aec.7003

ip address 192.168.30.254 255.255.255.0

standby 30 ip 192.168.30.1

standby 30 priority 110

standby 30 preempt

!

router ospf 1

log-adjacency-changes

network 192.168.10.0 0.0.0.255 area 0

network 192.168.1.0 0.0.0.255 area 0

network 192.168.20.0 0.0.0.255 area 0

network 192.168.30.0 0.0.0.255 area 0

!

ip classless

!

ip flow-export version 9

!

!

!

!

!

!

!

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

!

!

end

L3-Succursalle :

L3-Sucursalle>en

L3-Sucursalle#sh run

Building configuration...

Current configuration : 2566 bytes

!

version 12.2(37)SE1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname L3-Sucursalle

!

!

!

ip dhcp excluded-address 192.168.11.1

ip dhcp excluded-address 192.168.21.1

ip dhcp excluded-address 192.168.31.1

!

ip dhcp pool vlan10

network 192.168.11.0 255.255.255.0

default-router 192.168.11.1

dns-server 192.168.10.5

ip dhcp pool vlan20

network 192.168.21.0 255.255.255.0

default-router 192.168.21.1

dns-server 192.168.10.5

ip dhcp pool vlan30

network 192.168.31.0 255.255.255.0

default-router 192.168.31.1

dns-server 192.168.10.5

!

!

ip routing

!

!

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

!

interface FastEthernet0/2

switchport access vlan 20

switchport mode access

!

interface FastEthernet0/3

switchport access vlan 30

switchport mode access

!

interface FastEthernet0/4

no switchport

ip address 192.168.2.2 255.255.255.0

ip ospf 1 area 0

duplex auto

speed auto

!

interface FastEthernet0/5

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

switchport trunk allowed vlan 10,20,30

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan10

mac-address 0001.6413.7601

ip address 192.168.11.254 255.255.255.0

standby 10 ip 192.168.11.1

standby 10 priority 110

standby 10 preempt

!

interface Vlan20

mac-address 0001.6413.7602

ip address 192.168.21.254 255.255.255.0

standby 20 ip 192.168.21.1

standby 20 priority 110

standby 20 preempt

!

interface Vlan30

mac-address 0001.6413.7603

ip address 192.168.31.254 255.255.255.0

standby 30 ip 192.168.31.1

standby 30 priority 110

standby 30 preempt

!

router ospf 1

log-adjacency-changes

network 192.168.2.0 0.0.0.255 area 0

network 192.168.11.0 0.0.0.255 area 0

network 192.168.21.0 0.0.0.255 area 0

network 192.168.31.0 0.0.0.255 area 0

!

ip classless

!

ip flow-export version 9

!

!

!

!

!

!

!

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

!

!

end