ersion 15.6

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname Router

!

boot-start-marker

boot-end-marker

!

!

!

no aaa new-model

ethernet lmi ce

!

!

!

!

!

!

!

!

!

!

!

!

ip cef

no ipv6 cef

multilink bundle-name authenticated

!

!

!

!

license udi pid CISCO2911/K9 sn FCZ211840CK

!

!

!

redundancy

!

!

!

!

!

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface GigabitEthernet0/0

ip address 192.168.1.1 255.255.255.0

ip ospf 1 area 0

shutdown

duplex auto

speed auto

!

interface GigabitEthernet0/1

ip address 192.168.2.1 255.255.255.0

ip ospf 1 area 0

shutdown

duplex auto

speed auto

!

interface GigabitEthernet0/2

ip address 10.2.1.1 255.0.0.0

ip nat outside

ip virtual-reassembly in

duplex auto

speed auto

!

interface Serial0/0/0

no ip address

shutdown

!

interface Serial0/0/1

no ip address

shutdown

clock rate 2000000

!

router ospf 1

redistribute connected subnets

network 192.168.1.0 0.0.0.255 area 0

network 192.168.2.0 0.0.0.255 area 0

network 192.168.10.0 0.0.0.255 area 0

network 192.168.20.0 0.0.0.255 area 0

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip nat inside source list NAT_VLAN10 interface GigabitEthernet0/2 overload

ip nat inside source list NAT_VLAN20 interface GigabitEthernet0/2 overload

ip nat inside source list NAT_VLAN30 interface GigabitEthernet0/2 overload

ip nat inside source list NAT_VLAN40 interface GigabitEthernet0/2 overload

ip nat inside source list NAT_VLAN50 interface GigabitEthernet0/2 overload

ip nat inside source static tcp 192.168.10.3 80 interface GigabitEthernet0/2 80

ip route 0.0.0.0 0.0.0.0 10.0.0.1

!

ip access-list extended NAT_VLAN10

permit ip 192.168.10.0 0.0.0.255 any

ip access-list extended NAT_VLAN20

permit ip 192.168.20.0 0.0.0.255 any

ip access-list extended NAT_VLAN30

permit ip 192.168.30.0 0.0.0.255 any

ip access-list extended NAT_VLAN40

permit ip 192.168.40.0 0.0.0.255 any

permit ip 192.168.50.0 0.0.0.255 any

!

!

!

!

control-plane

!

!

!

line con 0

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport output pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

login

transport input none

!

scheduler allocate 20000 1000

!

end